LAB 03

MUL, DIV Instructions and CX/DX Registers

| Course | Computer Organization & Assembly Language |
| --- | --- |
| Lab No. | 03 |
| Topic | MUL, DIV Instructions and CX/DX Registers |

# 1. CX Register

CX is a 16-bit general-purpose register. It is also commonly used as a counter. It has two 8-bit parts:

CX = [ CH ][ CL ] Example: MOV CX, 1234H CX = 1234H CH = 12H CL = 34H

CX is not only a loop register; it can also store ordinary data.

MOV CX, 0005H ADD CX, 0003H

After execution, CX = 0008H.

# 2. DX Register

DX is a 16-bit general-purpose register. It has two 8-bit parts and has a special role in 16-bit multiplication and division.

DX = [ DH ][ DL ] Example: MOV DX, 5678H DX = 5678H DH = 56H DL = 78H

A useful beginner mental model: AX is the main working register; DX often works together with AX when a result or dividend is larger than one 16-bit register.

# 3. MUL Instruction

MOV AL, 05H MOV BL, 04H MUL BL

AL = 05H before MUL

BL = 04H

AX = 0014H after MUL

AH = 00H and AL = 14H after MUL

## 16-bit MUL

AX × 16-bit operand → DX:AX

## 32 bit MUL

DX:AX represents a 32-bit result. DX contains the high 16 bits and AX contains the low 16 bits.

MOV AX, 0010H MOV BX, 0005H MUL BX

16 × 5 = 80 decimal = 00000050H, so DX=0000H and AX=0050H.

# 4. DIV Instruction

AX ÷ 8-bit operand AL = quotient AH = remainder

## 8-Bit DIV

Example: 25 ÷ 4 = 6 remainder 1.

MOV AX, 0019H MOV BL, 04H DIV BL

## 16 Bit DIV

After DIV: AL=06H (quotient) and AH=01H (remainder).

DX:AX ÷ 16-bit operand AX = quotient DX = remainder

If your dividend is a simple value that fits in AX, clear DX first so DX:AX contains the intended dividend.

MOV DX, 0000H MOV AX, 0064H MOV BX, 000AH DIV BX

100 ÷ 10 = 10 remainder 0, so AX=000AH and DX=0000H.

# 5. MUL/DIV Quick Table

| Operation | Operand | Input | Result |
| --- | --- | --- | --- |
| 8-bit MUL | 8-bit | AL × operand | AX |
| 16-bit MUL | 16-bit | AX × operand | DX:AX |
| 8-bit DIV | 8-bit | AX ÷ operand | AL=quotient, AH=remainder |
| 16-bit DIV | 16-bit | DX:AX ÷ operand | AX=quotient, DX=remainder |

# 6. In Lab Tasks

## Program 1 — CX

MOV CX, 1234H

## Program 2 — DX

MOV DX, 5678H

## Program 3 — 8-bit MUL

MOV AL, 05H MOV BL, 04H MUL BL

## Program 5 — 8-bit DIV

MOV AX, 0019H MOV BL, 04H DIV BL

## Program 7 — 16-bit MUL

MOV AX, 0010H MOV BX, 0005H MUL BX

## Program 8 — 16-bit DIV

MOV AX, 0064H MOV BX, 000AH DIV BX

## Program 9 — 16-bit DIV with remainder

MOV AX, 0065H MOV BX, 000AH DIV BX

## Program 10 — CX + DX

MOV CX, 0005H MOV DX, 000AH ADD CX, DX

# 7. Practice Exercises

Multiply 07H by 03H using 8-bit MUL. Record AX.

Multiply 09H by 04H using 8-bit MUL. Record AX.

Divide 20 decimal by 3 using 8-bit DIV. Record quotient and remainder.

Divide 50 decimal by 7 using 8-bit DIV. Record quotient and remainder.

Put 4321H in CX. Find CH and CL.

Put ABCDH in DX. Find DH and DL.

Multiply 0012H by 0004H using 16-bit MUL. Record DX:AX.

Divide 0064H by 0008H using 16-bit DIV. Record AX and DX.